LIGO Laboratory / LIGO Scientific Collaboration

E1000389-v1	        LIGO                           07 Sept.  2010

aLIGO Sensor and Target Assembly Procedure

M.Hillard

Distribution of this document:

Advanced LIGO Project

This is an internal working note

of the LIGO Laboratory.

| California Institute of Technology LIGO Project – MS 18-34 1200 E. California Blvd. Pasadena, CA 91125 Phone (626) 395-2129 Fax (626) 304-9834 E-mail: info@ligo.caltech.edu | Massachusetts Institute of Technology LIGO Project – NW22-295 185 Albany St Cambridge, MA 02139 Phone (617) 253-4824 Fax (617) 253-7014 E-mail: info@ligo.mit.edu |
| --- | --- |
| LIGO Hanford Observatory P.O. Box 1970 Mail Stop S9-02 Richland WA 99352 Phone 509-372-8106 Fax 509-372-8137 | LIGO Livingston Observatory P.O. Box 940 Livingston, LA 70754 Phone 225-686-3100 Fax 225-686-7189 |

Clean room standards

For a clean assembly all LIGO standards should be followed, as presented in the latest version of the LIGO Contamination Control Plan (E0900047). Clean room garb including UHV gloves should be worn when working with parts.

All tools that come in contact with assembly should be cleaned to class B standards.

Assembly will be done under a portable clean room.  Any time a part of the assembly is not covered by the portable clean room or not being actively worked on it should be covered with appropriate clean covers. (C3 polyester or equivalent).

Torque Values for Bolts:

Standard torque values according to manufacturer standards for screw material should be used.

Parts prep work: alignment pins and helicoils

To facilitate assembly, parts requiring pins and or Helicoils may to be prepared before assembly and will be listed to allow preliminary preparation of the parts, after cleaning and before starting general assembly. See E1000402 Current Rev.

Helicoils and Dowel pin

1. D1000472

Weight

Approximated weights are provided in this assembly procedure as an indication for safety purpose.

Reference D1000468

1. Press Item (2) 1/2 x 3" lg dowel pin into Item (3) (D1000472) Position Sensor Target Body.

The dowel must be inserted carefully, if the

2. Be sure Items are clean and free of debris. Attach Item (1) D1000470 Position Sensor Target to Item (3) (D1000472) Position Sensor Target Body using Item (5) 8-32 x .50 SHCS and Items (4) vented washer.

Reference D1000467

1. Assemble Item (4) 3/8-16 x 2" lg. screw to Item (2) D1000469 Position sensor base and Item (3) D0902610 Sensor head spherical washer using Item (5) washer Item (7) spring washer and

Item (6) hex nut as shown does not have to be tightened this will be disassembled and reassembled at a later time.

2. Attach Item (1) Sensor to assembly using Item (8) washer and Item (9) m2x0.4 x 8mm lg screw. Be sure items are clean a free of debris before assembling.