LIGO Laboratory / LIGO Scientific Collaboration

T1300612-v1	LIGO	7/2/13

Figure measurement of 6” aLIGO Transmission Monitor Flats

GariLynn Billingsley

Distribution of this document:

LIGO Scientific Collaboration

This is an internal working note

of the LIGO Laboratory.

| California Institute of Technology LIGO Project – MS 18-34 1200 E. California Blvd. Pasadena, CA 91125 Phone (626) 395-2129 Fax (626) 304-9834 E-mail: info@ligo.caltech.edu | Massachusetts Institute of Technology LIGO Project – NW22-295 185 Albany St Cambridge, MA 02139 Phone (617) 253-4824 Fax (617) 253-7014 E-mail: info@ligo.mit.edu |
| --- | --- |
| LIGO Hanford Observatory P.O. Box 159 Richland WA 99352 Phone 509-372-8106 Fax 509-372-8137 | LIGO Livingston Observatory P.O. Box 940 Livingston, LA 70754 Phone 225-686-3100 Fax 225-686-7189 |

http://www.ligo.caltech.edu/

# Introduction

Two silver coated 6" x 1" flats were measured on the Zygo interferometer.  The data were analyzed over 144 mm and 100 mm apertures and found to be at worst lambda by 12 (at 633nm) over 144 mm aperture and at best lambda by 48 on 100mm aperture.

Data were taken at only one rotational angle, so absolute accuracy is roughly of the order ± 2nm.

## Results for “30 Arc Sec” flat

“30 arc sec” flat analyzed over 144 mm aperture.

PV: 52 nm, rms: 7.6  nm

“30 arc sec” flat analyzed over 100mm aperture.

PV: 13 nm,  rms: 2 nm

## Results for “Edmund-paper wrapped” flat

“Edmund paper wrapped” analyzed over 144 mm aperture: PV: 36.9 nm, rms: 7.3 nm

“Edmund paper wrapped” analyzed over 144 mm aperture: PV: 17 nm, rms: 3.8 nm