LASER INTERFEROMETER GRAVITATIONAL WAVE OBSERVATORY

-LIGO-

CALIFORNIA INSTITUTE OF TECHNOLOGY

MASSACHUSETTS INSTITUTE OF TECHNOLOGY

| Document Type | DCC Number T1300888-v1 | October 23, 2013 |
| --- | --- | --- |
| TCS Power Supply Splitter Test Plan |  |  |
| B. Abbott |  |  |

Distribution of this draft:

This is an internal working note of the LIGO Laboratory

California Institute of Technology	Massachusetts Institute of Technology

LIGO Project – MS 18-33	LIGO Project – MS 20B-145

Pasadena, CA 91125	Cambridge, MA 01239

Phone (626) 395-2129	Phone (617) 253-4824

Fax (626) 304-9834	Fax (617) 253-7014

E-mail: info@ligo.caltech.edu	E-mail: info@ligo.mit.edu

www:  http://www.ligo.caltech.edu/

# Introduction

The tests described below are required to verify the correct pinout and continuity of the TCS Power Supply Splitter (D1300680).

# Test Equipment

DMM

Two 9-pin Dsub Breakout Board

# Tests

## Continuity

Plug a 9-pin breakout onto the appropriate connectors below, and check the signal continuity using a DMM.  The splitter box fails the test if any of the connections in the table are broken.

| Channel Name | Pins on DB9 “From Power Supply” | Pins on DB9 “To AA Chassis” | Pins on DB9 “To Interlock” | Connected? Y/N |
| --- | --- | --- | --- | --- |
| VMon | J1 Pin 1 | J2 pin 1 |  |  |
| IMon | J1 Pin 6 | J2 pin 2 |  |  |
| GND | J1 Pin 2 | J2 pin 6 |  |  |
| GND | J1 Pin 2 | J2 pin 7 |  |  |
| Remote + | J1 Pin 4 |  | J3 pin 1 |  |
| Remote - | J1 Pin 9 |  | J3 pin 6 |  |