A Preliminary Study of the Vacuum System for the Beamtubes of the Cosmic Explorer Third Generation Gravitational Wave Detector

R.Weiss  January 20, 2019, June 6, 2019 Includes changes and suggestions coming from Vacuum Workshop

NSF Vacuum Workshop at the LIGO site in Livingston Louisiana January 29 to 31, 2019

ABSTRACT  A concept study for the beamtube vacuum system of a third generation  gravitational wave detector with 40km arms is presented. The concept is a derivative from the successful design and operation of the current 4km detectors but with the intent of reducing the initial construction costs per km as well as (hopefully) the running and maintenance costs. The residual gas pressure requirements are more stringent than those for the initial detectors, they are close to the goal values which were actually achieved in the initial detectors.

The concept trades on the idea that there are engineering and operational advantages to separating the functions of maintaining a space against the atmospheric pressure load and reducing the residual gas to the levels of ultra high vacuum. The concept is to use a nested vacuum system with an outer shell of extruded standard steel tubing with an inner tube of thin wall aluminum. The system is divided into 10km long modules which can be evacuated independently. The modules are further divided into 2km long sub-modules isolated by low differential force valves (“soft “ close valves). Each 2km sub-module includes an annular valve that separates the inner and outer system (again a “soft” closed valve structure).

The nested system has more tolerance to leaks and weld workmanship than the standard vacuum system and may even have advantages in reduced diagnostic time and maintenance costs. However, the assembly costs if not done with good engineering design could eliminate the cost advantages. The assembly and installation, especially of the inner system, would benefit from special tooling designed for in-field manufacture and assembly.

The study includes model pumping curves for a specific configuration that meets the scientific requirements. The model shows that even for a nested system the most significant vacuum problem is the control of water which sets the costs and the pump down times. An investment in reducing the adsorbed water prior to pump down would pay off handsomely.

Introduction

One of the most direct ways to increase the sensitivity of interferometric gravitational wave detectors and bring the new field of gravitational wave astronomy into cosmology is to increase the arm length of the detectors. The gravitational wave signal increases as the arm length while many of the competing noise terms are length independent or grow only slowly with the length.  A detector with 40km arms, a factor of 10 increase over the first generation gravitational wave detectors that have open the field, will allow measurements of compact binary sources such as black holes and neutron stars to the epoch of the first formation of stars in the universe.

The main challenge is the increased cost of the arms which grows somewhat faster than linear in the length. The cost is tied up with construction of the vacuum system – tubes, pumps, valves, outgassing, surface preparation, vacuum hardware and fittings, control of corrosion - and the civil construction – alignment, berms, cut and fill or tunneling, enclosures, roads, power distribution. Some operations costs will also grow with length but are not studied here except to note the need to leak hunt and maintain the system against corrosion, power costs to maintain the vacuum and security need to be evaluated

In the appendix on costs, the beamtube costs for the two 4km LIGO detectors constructed in 1994 dollars is shown to be $74.6M . The length dependent costs for the 16km of beamtube was $56.8M about $3.55M/km. A straightforward extrapolation for 80km of comparable beamtube constructed in 2025 using a 2.3% per year inflation  would give a total cost of $603M and a length dependent cost of $7.1M/km. The aim of the workshop is to establish if there are practical ideas to reduce this cost and still satisfy the scientific and engineering requirements for the beamtubes. It is worth noting that natural gas pipelines of the same diameter and length are estimated to have a total cost of  $270M and a length dependent cost of $3.38M/km. A preliminary estimate using the concept described in this note gives a total cost of $380M and a length dependent cost of 4.47M/km.

The concept explored in this note is to look at a beamtube system in which the structural demands to withstand the atmospheric pressure and simultaneously the requirements to maintain ultrahigh vacuum conditions are satisfied by a nested system – an outer tube of commercial material such as ordinary steel or a composite and an inner tube with low outgassing properties such as thin wall sheet aluminum, see Figure 1. The nesting relaxes the demands on both tubes. It allows cost savings in both material and in the vacuum components as well as in the procedures to attain the partial pressures and other system requirements such as cleanliness and optical properties. The nesting also enables optimization of the pumping system to achieve the pressure requirements. For example, it allows use of getter pumps which have a finite total gas capacity to maintain the inner UHV system.  However, a cost driver could be the additional complexity in assembly.

Vacuum requirements for the 40km beamtubes

The primary requirement is on the column density of the gas (average pressure over the length of the tube). The pressure in the system is low enough everywhere to be in mean molecular flow. The principal issues in the beam tubes are the phase fluctuations in the optical beam due to forward scattering by the residual gas (refractive index fluctuations). The largest price is paid for molecules with a large optical polarizability (large molecular volume) and which have low thermal velocities  (large masses which have longer residence time in the optical beam). Atomic hydrogen is one the least perturbative while a heavy hydrocarbon is the most damaging.

The noise power spectrum of the gravitational wave strain due to gas column density fluctuations is given by:

Table 1 The average pressure requirements for a 40km detector with a sensitivity of h(f) ~1 x 10-25 strain/ at 300K and an optical  wavelength of 1 micron.

| Gas species | ratio to H2 | 40km req torr | Gas species | ratio to H2 | 40km req torr |
| --- | --- | --- | --- | --- | --- |
| He | 0.32 | 9.8 x 10-9 | Kr | 8.27 | 1.4 x 10-11 |
| Ne | 0.89 | 1.3 x 10-9 | Xe | 14.9 | 4.5 x 10-12 |
| H2 | 1.0 | 1.0 x 10-9 | AMU 100HnCm | 38.4 | 7.0 x 10-13 |
| H2O | 3.3 | 1.1 x 10-10 | AMU 200HnCm | 88.8 | 1.4 x 10-13 |
| N2 | 4.2 | 6.5 x 10-11 | AMU 300HnCm | 146 | 5.0 x 10-14 |
| A | 4.51 | 4.9 x 10-11 | AMU 400HnCm | 208 | 2.5 x 10-14 |
| CO | 4.6 | 5.0 x 10-11 | AMU 500HnCm | 277 | 1.4 x 10-14 |
| CH4 | 5.4 | 3.0 x 10-11 | AMU 600HnCm | 345 | 9.0 x 10-15 |
| CO2 | 7.1 | 2.3 x 10-11 |  |  |  |

Note: Pressure requirement are set so that the noise from any single gas species can be no larger than 1/3 of the allowed strain noise.  “ratio to H2” means   for the gas relative to that of Hydrogen.

Summary of the design concept  including other system requirements

Table 2 Design concept and other requirements

| Concept |  |  |
| --- | --- | --- |
|  | geometry | 40km arm is broken up into four independent 10km modules with gate valves able to withstand atmospheric loads at each end. The inner system modules are further broken up into five 2km sub-modules with separating gate valves between them that do not withstand atmospheric loads (“soft close gate valves”). Inner and outer systems in each sub-module are separated by “soft” close annular valves. See Figure 2 and 3. |
| Vacuum |  |  |
|  | leaks assumed in the model | outer system -> inner system: F =1 liter/sec/km atmosphere -> outer system: F =10-7 liters/sec/km, Q = 10-4 torr liter/sec/km |
|  | pumps in 10km module | Rough pumping: Blowers and turbos on movable trailers to be used on all the10km modules at the ends and the middle. Permanently mounted pumps: Outer system : 2000 liter/sec ion pump mounted at the middle of each 2km sub-module. Inner system: 500 liter/sec ion pump mounted at the middle of each 2km module. ZAO getter pumps distributed at 250 meter intervals inside the inner system. See Table 3 of ZAO properties and Figure 2 and 3 |
|  | valves in 10km module | Two standard hard closed gate valves one at each end of a 10km module. Four “soft” close separator valves between the 2km sub-modules. Five “soft” close annular separator valves Figure 4 between the inner and outer systems in the middle of each 2km sub-module |
|  | pump out | Inner and outer systems rough pumped together. Carried out on 10km modules with pumps on the ends and the middle of the module. At 10-4 torr begin 360K bakeout of inner system. At 30days with 3x10-5 torr, start outer system 2000 liter/sec ion pumps on each 2km sub-module. At 53 days 3 x 10-6 torr, activate ZAO getters, after pressure surge close the “soft” close annular separator valves on each 2km module. At 10-6 torr in the inner system start the 500 liter/sec ion pumps. At 140 days (depends on the initial adsorbed water load) turn off the bakeout power. For model pressure curves vs time see Figure 5 and 6. |
|  | bakeout | Assume initial 150 monolayers of water on surfaces of both inner and outer system with a Dubinin –Raduskevich (DR) skewed Gaussian adsorption peak of 104K and a repulsive potential factor of r=0.7. No bakeout of outer system, a 360K bakeout of the inner system for 3 months. Power requirement 30kW/km with two radiation shields between inner and outer system. Bakeout done in all 2km sub-modules of a 10km module simultaneously. See Figure 3 |
|  | pump down time | Several months primarily due to water desorption. Can be significantly speeded up by reducing the initial adsorbed water . |
|  | reliability | 20 year lifetime. Ability to maintain and service: fully isolatable 10Km modules and at low pressure < 10-3 torr isolatable 2km sub-modules. Enables diagnostics for leak hunting. |
|  | contamination | < 1/10 monolayer/month of hydrocarbon deposition from the beamtube onto the optics at the ends of the 40km arms |
|  | dust | Less than 1 one micron “diameter” dust particle falling through the optical beam/km/hour. Needs further study. |
| Mechanical and optical |  |  |
|  | clear aperture | current design is 110cm diameter; needs more study for 1 micron light and, especially, with use of 2 micron light in the interferometer in a later phase |
|  | scattering and diffraction from walls and baffling | @ 1 micron wavelength reflectivity of wall < 0.1 at 450 , BRDF(>450 )<10-3 /steradian randomly serrated edge baffles mounted in inner tube. Needs more study with use of 2micron light. |
|  | vibration | inner tube mounted to move no more than twice seismic motions above 5Hz. Baffles mounted with f > 5Hz isolators; both need more study. |
| Temperature |  |  |
|  |  | inner system needs to accommodate a low temperature bake upto 360K, outer system needs to accommodate 260K to 330K depending on location. Inner system has relief V expansion bends every 2 meters, outer system has 5 bellows expansion joints/2km |
| Safety |  |  |
|  | maximum pressure difference inner/outer system | A concern expressed at the workshop was the crushing of the inner system by excess pressure in the outer system. Using the stability equation in the appendix, the maximum excess pressure in the outer system over the inner one is about 0.1 torr for 0.5mm inner wall thickness of aluminum. The allowed pressure difference grows as thickness cubed. The there needs to be an automatic opening system for the annular valves in the case of a sudden large leak to the outer system. |

Getter cartridge needs

to be mounted on

thicker surface such as

5mm thick cylinder

to take the activation

power

Figure 1 Schematic of the nested system. The outer tube is extruded standard  48” OD steel tubing . Two concepts are being carried, one uses 3/16” wall with periodic stiffening rings on the outside to avoid collapse. The other is to use 3/8” wall, the standard size for the natural gas industry, without stiffening rings. Later in the design, depending on the site, the thicker wall may have a cost advantage as it allows less secure enclosures for the tube (more economical, more easily punctured by bullets). The hydrogen outgassing for the outer system at 300K is assumed to be 10-11 torr liters/sec/cm2 and the water load at construction is assumed as 150 monolayers

(5 x 10-3 torr liters/cm2). The inner system is composed of aluminum sheet ½ mm thick. It is spiral roll bonded (or by other cold welding techniques) in a continuous helix at the detector assembly site. A thermal relief V is rolled into the tube every 2 meters. The V helps to stiffen the inner tube but also provides places to mount serrated edge baffles to reduce diffracted propagation of the laser light in the tube. The V also provides places to hide ZAO getter pumps and wiring to avoid scattering of the laser light. The inner wall of the aluminum needs to have low reflectivity for 1 and 2 micron light. The outer wall of the aluminum tube should remain a low emissivity surface but be insulated electrically to avoid electrical shorts to the outer structure. The water loading is expected to be the same as for the outer system but the hydrogen outgassing is expected to be less than

10-14 torr liters/sec/cm2. During the pumpdown the inner aluminum tube is heated to 360K by passing 1000 amperes directly through the tube to reduce the adsorbed water. Two aluminum radiation shields both 1/2 mm thick (or sheets of super insulation) are placed between the inner aluminum system and the outer steel tubes.

SCHEMATIC OF A 10 KM MODULE

Figure 2 Schematic of a 10km module of the beamtube. For ease of construction, testing, commissioning and service; it is necessary to divide the arms into independent modules. The concept uses 10km modules which can be pumped down independently. Full hard close gate valves, able to withstand atmospheric loads, are placed at the ends of the 10km module. The 10km module is further divided into five 2km sub-modules. The 2km sub-modules can be isolated from each other by “soft” close valves which are not designed to take atmospheric loads. The “soft” close valves maintain isolation between the inner and outer systems. These valves are useful in making operations  and diagnostics easier . For example, one does not have to try to get UHV conditions in all inner system sub-modules simultaneously  (note: if these valves are difficult to design and costly one could forgo them). After rough pumping the entire module (both inner and outer systems), the outer system is maintained by 2000 liter/sec ion pumps placed at the center of each 2km sub-module. The inner system is heated to 360K when the pressure has reached 10-4 torr (see figure 3).  The inner and outer systems are isolated by “soft” close annular valves. The “soft” close annular valves are open during the pump out until the pressure in the system has dropped to 3 x 10-6 torr. At this pressure the ZAO getters are activated followed by the closing of the “soft” close annular valves. The  pump speed and capacity of the getter for the different residual gas species is described in Table 3. The getter is distributed in the inner tube at intervals of 250 meters with a total getter mass of 12kg per 2km module. The 500 liter/sec ion pumps that pump the inner system are turned on. The pressure vs time of the outer system is shown in Figure 5 and that of the inner system in Figure 6.

Figure 3 Schematic of a 2km sub-module.  The figure shows the concept for the bakeout at 360K. Contact to the circumference of the inner system aluminum tube is made at the valves. The “soft” close valves are grounded while the annular valve is floating. DC power supplies deliver 1000A at 30V to the inner aluminum tube in 1km long circuits.

After the picture was drawn it became apparent that one could use the outer tube as the current return as it will most likely be a 3/8” wall steel tube without insulation on it. (Do not need the long external cables). Radiation shields composed of 2 sheets of thin wall aluminum (or superinsulation) reduce the power requirement to attain the bakeout temperature. Figure 4 shows a concept for the “soft” close annular valves. The outer wall of the inner system aluminum tube needs to be coated with an electric insulator that does not increase the infrared emissivity. The inner wall needs to be coated with a low reflection surface for 1 micron to 2 micron wavelength radiation to absorb the scattered laser light.

Figure 4a Soft close inner/outer and hard close outer valve concept

Figure 4b Soft close annular inner/outer valve concept.

Figure 4  Concepts for the “soft” close annular valves and shutter. The pumping speed between the inner and outer system when open should be greater than 6000 liters/sec. The leakage through the valve when closed should be less than 1 liter/sec. No lubrication or vacuum grease is permitted in the valve.

Pump down

Figure 5  Pumpdown of the 10km module outer system

Figure 6 Pumpdown of the 2km sub-module inner system

The pump down curves were calculated with a finite element program that included the diffusion in the long tubes and the outgassing by the walls as a function of temperature. Water pressure is determined using the Dubinin-Raduskevich adsorption theory with emission and readsorption. Leaks between the inner and outer systems as well as leaks of the outer system to the external atmosphere are included. The symbolic equations below (valid only for a steady state) show the various terms in the model for each gas species indicated by the index k.

Pumping

Outer system 10km module: five 2000 liters/sec ion pumps

Inner system  2km module:  12kg SAO getter@473K distributed in 250meter intervals, properties given in Table 3, one 500 liters/sec ion pump

Outgassing

Outer system

Hydrogen: J(H2, 300K) = 1 x 10-11 torr liters/sec cm2

Water:  Dubinin –Raduskevich (DR) 150 monolayers T0 =10000K, r= 0.7

Inner system

Hydrogen: J(H2,300K) = 1 x 10-14 torr liters/sec cm2

Water:  Dubinin –Raduskevich (DR) 150 monolayers T0 =10000K, r= 0.7

Leaks

Finner-outer   1 liter/sec/km

Fouter-external  10-7 liter/sec /km

Tube

outer:  radius= 62cm   10km module:  area=3.9x108 cm2 , volume = 1.2x107 liters

inner:  radius= 59cm     2km sub-module: area = 7.4x107, volume = 2.2x106 liters

V thermal relief on inner tube: height from inner wall = 6cm

Table 3 Getter properties

Properties assumed for 12kg of ZAO getters held at 473K in the 2km sub modules

| gas species | lit/s/gm | torrlit/gm | torr lit 2km | lit/s 2km | Preq torr | sat yr |
| --- | --- | --- | --- | --- | --- | --- |
| H2 | 2 | 15 | 1.8x105 | 2.4x104 | 1.0x10-9 | 240 |
| H2O | 2(assume) | 15(assume) | 1.8x105 | 2.4x104 | 1.1x10-10 | 2400 |
| N2 | 0.43 | 2.14 | 2.6x104 | 5.2x103 | 6.5x10-11 | 2650 |
| CO2 | 0.46 | 0.17 | 2.0x103 | 5.5x103 | 2.3x10-11 | 570 |
| O2 | 0.8 | 12.9 | 1.5x105 | 9.6x103 | 5.0x10-11 | 9930 |
| nobel gases | 0 | 0 | 0 | 0 |  |  |

ZAO  are not regenerated at 400C

Note for costs: Assume ZAO cost $15/gm in large quantities which corresponds to $180K/2km sub module and $7.2M/40km detector (2025yr dollars)

Pumpout procedure

The pumpout described is not optimized in terms of the time required nor the number of pumps used. This optimization is work for a real engineering study and will involve tradeoffs between the allowed leaks, the number and location of the pumps and pumpdown time.

The procedure used in the Figures 5 and 6 is presented here.

Rough down in 10km modules at both ends and in the middle with portable roughing pumps and turbos; in the models assumed 2000 liters/sec at each of the three ports. Begin with the “soft” close gate valves and the “soft” close annular pumps open. All ion pumps are off and getters not activated. At day 30 the water pressure is 3 x 10-5  torr . Remove the portable roughing pumps and begin ion pumping with the 2000 liter/sec pumps on each 2km sub-module. At day 37 begin heating the inner tube to 360K. At day 53 activate the ZAO getters. After the surge of gas has pumped away close the “soft” close annular valves and the “soft” close gate valves between the 2km sub-modules. Turn on the 500 liter/sec ion pumps on the inner system. At this point there are 16 monolayers of water left on the inner system walls. On day 140 stop the heating of the inner tube. When the system returns to 300K the pressures satisfy the requirements. The adsorbed water on the inner surface is 12 monolayers.  The equivalent of 4 monolayers have been taken up by the ZAO getter (104 torr liter/2km of the inner tube which has 12kg of ZAO, a water load of about 1 torr liter/gm of ZAO ) If the ratio of pumping speed to stored water could be increased, the amount of ZAO getter needed could be reduced. Another finding is that the pump down time is dominated by the readsorption of the water. The pump down time is close to proportional to the initial water loading on the surface and, with these model parameters, is still effected by the pumping speed,  not only by the outgassing rate.

Appendices: calculations, other findings, preliminary costs

Pressure distributions along the modules for various conditions

Figure  7 Water pressure distribution in a 2km sub-module for a 2liter/sec leak between the inner and outer systems.

Figure 8 Nitrogen pressure distribution as a function of pumping strategy in a 10km module.

Testing the bakeout strategy

Figure 9 Bakeout to eliminate water for different bakeout times and temperatures.

All sample bakes are for initial surface adsorption of 150 monolayers, have the same DR parameters and the same system pumping speed. The bake begins at 10 days after the start of pumping and return to 300K at the downward discontinuity in the 1/t dependence. The results show the longer the bake and the higher the bake temperature the lower is the final water pressure when the system returns to 300K. Runs were made with initial water load of 15 monolayers of adsorbed water (curves are not shown). These were all about a factor 10 lower than the curves plotted.

Pumping speed of 1km of 62 cm radius beamtube for different gases

Useful for quick estimates to determine if pump speed or the beamtube diffusion limits the pressure.

Table 4 Pumping speed of 1km long 62cm radius tube at 300K

| gas | amu | v thermal | F liters/sec |
| --- | --- | --- | --- |
| H2 | 2 | 1.8 x105 | 910 |
| H2O | 18 | 5.9 x 104 | 295 |
| Ne | 20 | 5.6 x 104 | 280 |
| N2 | 28 | 4.7 x 104 | 240 |
| A | 40 | 4.0 x 104 | 200 |
| CO2 | 44 | 3.8 x 104 | 190 |
| Kr | 84 | 2.7 x 104 | 135 |
| Xe | 132 | 2.2 x 104 | 108 |

Tube diameter and wall thickness for stability against crushing,  no stiffeners

Table 5 Tube wall thickness , diameter and mass/ kilometer for 2 atmospheres crushing pressure: material mass/length and cost/length

| material | thickness/diameter | diameter inches,cm | thickness inches,cm | kg/km | 2025yr $/km |
| --- | --- | --- | --- | --- | --- |
| S steel with stiffeners initial LIGO | stiffeners 1.3X103/km 4.45x0.47cm2 | 48”, 122cm | 0.13”, 0.3 cm | 9.96 x 104 | 0.352M |
| steel with stiffeners | stiffeners 1.3X103/km 4.45x0.47cm2 | 48”,122cm | 0.13”, 0.3cm | 9.96 x 104 | 0.120M |
| tube steel | 7.717 x 10-3 | 48” ,122cm | 0.37”, 0.939cm | 2.81 x 105 | 0.338M |
| tube steel |  | 60” ,152cm | 0.46”, 1.17cm | 4.36 x 105 | 0.524M |
| aluminum | 1.094 x 10-2 | 48” ,122cm | 0.53”, 1.33cm | 1.37 x 105 | 0.563 |

Note: Commerically formed tubing typically costs 2.2 times the material costs.

Bakeout parameters

For pressure less than 10-4 torr the thermal conductivity is close to being only radiative. The power radiated between an inner surface at temperature T2 with infrared emissivity  and an outer surface  at temperature T1 with infrared emissivity   separated by N radiation shields with infrared emissivity  is given by

Table 6 Power required to heat aluminum inner tube for a variety of temperatures.

T1 = 300K. , radius of the aluminum tube 59cm, thickness 0.5mm, electrical resistivity of aluminum at 300K is 3 x 10-8 ohm meter.

| T2 K | P/A w/m2 N=0 | V volts/km N=0 | A amp N=0 | P/A w/m2 N=2 | V volts/km N=2 | A amp N=2 |
| --- | --- | --- | --- | --- | --- | --- |
| 330 | 10.7 | 25 | 1577 | 2.2 | 11 | 715 |
| 340 | 15 | 30 | 1872 | 3.1 | 13 | 840 |
| 350 | 19.7 | 34 | 2147 | 4.0 | 15 | 970 |
| 360 | 25 | 38 | 2418 | 5.1 | 18 | 1070 |

Extrapolation of costs from 4km LIGO to 40km Cosmic Explorer and estimates for natural gas lines with comparable tube lengths and sizes

Table 7 LIGO beamtube costs  1994 dollars

References: LIGO Cost Book, L.Jones notes, F.Assiri notes

| item | total cost $M | length dep cost $M |
| --- | --- | --- |
| Management | 5.5 | ~1.0 |
| Design | 0.83 |  |
| Tube factory | 8.05 |  |
| Tube material | 9.02 | 9.02 |
| Bellows | 0.77 | 0.77 |
| Assembly | 1.66 | 1.66 |
| Leak check | 0.87 | 0.87 |
| Anchors+supports | 3.05 | 3.05 |
| Pump ports | 1.83 | 1.83 |
| Handling/shipping | 0.92 | 0.92 |
| Moblization/demobilization | 0.46 |  |
| Installation in field | 7.2 | 7.2 |
| Insulation and bake | 7.0 | 7.0 |
| Acceptance tests | 1.1 |  |
| CB&I profit (10%) | 5.2 | 3.3 |
| Beamtube enclosure design | 0.64 |  |
| BTE QA and survey | 0.32 | 0.32 |
| BTE, berm, slab construction | 18.6 | 18.6 |
| TOTAL 1 | 72.4 16.8 fixed | 55.5 3.47/km |
| Vac equipment for beamtube |  |  |
| gate valves | 0.72 | 0.72 |
| ion pumps | 0.24 | 0.24 |
| cryo traps | 0.80 |  |
| roughing pumps | 0.24 | 0.16 |
| turbo pumps | 0.18 | 0.12 |
| gauges, electronics | 0.02 | 0.02 |
| Total vac equip for beamtube | 2.2 0.94 fixed | 1.26 0.08/km |
| TOTAL 2 | 74.6 17.7 fixed | 56.8 3.55/km |

Projection for a single 40km interferometer in 2025 dollars using an inflation rate of 0.023/year, a factor of 2. Fixed costs = $35M, length dependent costs = $7.1M/km. Total cost for 80km of arm length = $603M

Note: The vacuum equipment costs for all of LIGO were not part of the beamtube contract. The fraction of the costs associated with the beam tube and have been estimated by me.

Costs of a 40km interferometer based on industry experience with natural gas pipelines

Standard schedule cold rolled steel tubing with corrosion resistant covering used in natural gas pipelines in sizes 24” and larger has a 3/8” wall. The total costs for pipeline installation and construction using shallow burial in “normal” sites was $60/ft3 in 2011 dollars. Large fluctuations in installed pipeline costs are common see Figure 10 .

Table 8 :The costs breakdown for natural gas pipelines

| item | fraction of total |
| --- | --- |
| Labor | 0.4 |
| Material | 0.31 |
| Survey,engineering,freight taxes | 0.23 |
| right of way costs | 0.07 |

Using the inflation rate of 0.023/year, the estimated total cost in 2025 dollars for installed 48” diameter tube/km is $3.38M/km. Total costs for 80km of tube length are about $270M .

Figure 10 Annual Average cost of pipeline elements in $/ft3 installed.

Reference “Historical Pipeline Construction Cost Analysis”  Z. Rui, P.A. Metz, D. Reynolds, G. Chen, X. Zhou ; International Journal of Oil, Gas and Coal Technology V4 #3 244 (2011) .

Cost estimate and discussion

Table 9 Preliminary 2025 costs and discussion

| item | cost/ | # | cost | discussion |
| --- | --- | --- | --- | --- |
| outer tube | 0.64M/km | 80km | 51M | Extruded 48” diameter standard steel tubing 3/8” wall to stop (most ) bullets and take atmospheric load with out stiffening rings. Fabricated tube 2.2* material cost which is $0.45/lb |
| expansion bellows | 15K/bellows | 5/2km | 3M | LIGO cost x inflation. Fewer needed /km since the bakeout does not heat the outer tube and is lower temperature |
| inner tube | 0.02M/km | 80km | 1.6M | Aluminum sheet 0.5mm thick made into tubing at the site with thermal relief V. Material cost only |
| in field machine to fabricate tubing |  |  | 3M | Forms tube by spiral roll welding, bends V thermal relief, prepare inner surface for low reflectivity and outer surface with electrical insulator, possibly reduce the adsorbed water |
| radiation shields | 0.04M/km | 80km | 3.2M | Aluminum sheet 0.5mm thick formed into cylinders or superinsulation (need to establish the water adsorption of the superinsulation) |
| Anchors @supports | 0.37M/km |  | 30M | LIGO cost x inflation |
| pump ports | 10K/port | 1/km | 0.8M | Ken Mason estimate |
| bake out | $0.1/kwh $3K/supply | 5.8Gwh 10 sup. | 0.57M 0.03M | 10km modules (5 sub-modules) heated simultaneously, circuit consists of aluminum inner tube and current return through the outer steel tube. Costs are 10 power supplies to be moved between the modules and the power 30kw/km for 100 days |
| installation |  |  | 86M | LIGO cost * inflation * 1.2 (increased complexity) |
| road 3m | 0.161M/km | 80km | 13M | asphalt road at $5/ft2 |
| berm 4m | 0.301M/km | 80km | 24M | concrete berm 6” thick at $7/ft2 |
| cut & fill 4m |  |  | <42M | Site dependent. Assume flat (curved with earth radius) site, vol.cut = vol.fill, 30 degree slope on side walls. midpoint of tube 21 meters below ground with ends 12 meters above ground on a trapezoidal berm. Horizontal base of the trapezoid 4m. Needs optimization to the site contours. A bowl with the right curvature could greatly reduce the cost. Estimate earth moving cost $7/m3 . |
| cover 3m | 0.45M/km | 80km | 36M | 8 ft high steel Quonset hut along berm. Assume outer tube is 3/8” steel wall able to stop hunter’s bullets |
| getter pumps | 12kg/2km $15/gm |  | 7.2M | ZAO cartridges, cost projected for large purchase |
| ion pumps inner | 10K/pump | 1/2km | 0.4M | 500 liter/sec |
| ion pumps outer | 25K/pump | 1/2km | 1M | 2000 liter/sec |
| gate valves | 100K/GV | 2/10km module | 1.6M | VAT valve or equivalent |
| soft close GV | 40K/GV | 5/10km | 1.6M | guess |
| annular GV | 20k/GV | 5/10km | 0.8M | guess |
| leak check | 0.1K/10m | 8000 sections | 0.8M | look for bubbles in each steel tube section, 10 m lengths |
| handling and shipping |  |  | 9M | LIGO cost x inflation |
| Quality assurance and survey |  |  | 3.2M | LIGO cost x inflation |
| Length dep management |  |  | 10M | LIGO cost x inflation |
| Length independent costs | 15.8M |  |  | LIGO cost * inflation for those items that remain in the concept |

The cost is $346M before profit. With 10% profit for the contractor, the total cost is $380M. The length dependent cost is 4.57M/km

Tunneling rather than cut and fill construction

Given the large cost associated with cut and fill and the even larger costs for excavation, it is worth looking at the current cost of tunneling. When LIGO was first being considered tunneling in the United States was far behind the rest of the world technically and correspondingly extremely expensive. It has not changed much though a company started by Elan Musk, The Boring Co, to build underground transportation is trying to make a significant change. Table 10 shows the tunneling costs delivered and projected by The Boring Co as well as the results from a paper “Assessing and Bench Marking the Construction Cost of Tunnels” by A. Bernardos delivered at a GEOPhysics meeting in Montreal in 2013.

TABLE 10  Tunneling costs

| method | $/m3 | $/km 4m diameter | $/80km |
| --- | --- | --- | --- |
| cut@fill | 9 7 | 0.1M | 8M |
| trapezoidal cut&fill ,equal volumes cut and fill (costed in Table 9) | 9 7 | depends on site topography | <42M |
| excavation | 16 9 | 0.2M | 16M |
| TBC current | 440 | 5.5M | 440M |
| TBC projected mid 2020s | 220 | 2.8M | 224M |
| Mean Europe (Bernardos) | 180 | 2.2M | 176M |

Tunneling looks unpromising except for short lengths where the earth moving may have peak cost. The accounting of cut and fill is not the same as in Table 9 where the trapezoidal shape of the structure required to maintain stability of cut channels and built up berms is accounted for. There is a 33meter height difference between the center of the beamtube and the ends 20 km away. This is due to the Earth’s curvature.

Why is the extrapolated initial LIGO cost for cosmic explorer different than the preliminary estimated cost?

Items that are less costly/km

1) The use of a quonset hut and thicker steel tubing rather than the concrete enclosure

2) Use of commercially fabricated steel with less stringent requirements on hydrogen outgassing

3) Standard steel tubing rather than stainless steel

4) Reduced cleaning requirements of the outer system and easier to attain cleanliness of the inner system

5) Bakeout costs are smaller: no insulation , lower tempertures and power costs

6) Reduced leak requirements on fabrication and less sensitive testing needed

7) Reduced cleaning

8) Reduced number of pumpout ports due to internal getter pumps

9) Improved match of pumps to requirements

9) May have left out some critical costs

Items that are more costly/km

1)  Assembly of the nested system (assumed 20% increase of infield assembly costs)

2) Use of getter pumps

3) Use of “soft” close longitudinal and annular gate valves

Topics for further work and tests to do

Gas bursts: Is a nested system more likely to make short ( 1 to 0.01 sec ) gas bursts that would be confused with gravitational wave signals?

“Soft” close valves: Possible concepts for the annular soft close valves and shutters as well as the combined hard close valves with additional annular valves are shown in the Figure 4a and 4b. Need to put more work into an actual design. The costing assumes  $100K for each hard closed valve, $40K for the “soft” closed and  $20K for the annular ”soft” close valves

Pump down time and costs: Run more cases to optimize the pump down time and costs. Determine the scaling for the adjustable parameters: the water load, the number of pumps, their location and the size of the  leaks. Most critical is probably pumping out the water as it recombines with the surface.

Water: Adsorbed water is a major cost driver in both the nested system as well as in conventional designs with a single tube. Research on reducing the adsorption is well worth the effort. Some examples: non adsorbing coatings, removal by UV of sufficient energy to break up the water into its atomic components, microwave discharge, dry purge surface scrubbing systems…….

A useful idea might be to rid the surface of moderately bound water, water that at 300K requires longer than weeks to be emitted. This would change the 1/t depends to a faster rate. It is not as crazy as one might think, tightly bound water, say with an emission rate of a century or longer does not contribute to the outgassing rate (in part also because of the reduced number of high binding energy sites). The repopulation rate of the sites depends on the density of water molecules hitting the surface but also inversely on the adsorption binding energy.

We need to test the idea of recovering from a vacuum disaster with a backfill very low in water to avoid having to rebake. This was part of a vacuum recovery and research proposal made to the NSF by LIGO but which was only partially funded.  Jon Feicht, LIGO vacuum engineer can tell about the planned research and test. Getting this right is critical for current LIGO due its problems with the gate valves and the stainless steel corrosion and will affect decisions that need to be made for 3rd generation detectors.

Getter tests: An assumption made in the concept is that an unsaturated ZAO getter does not outgas water. The outgassing of a getter as a function of accumulated water needs to be measured.

Optical properties: A problem specific to interferometric gravitational wave detectors is to avoid scattered and diffracted light from being recombined with the main optical beam sensing the gravitational wave. Besides baffling to avoid multiple bounce paths from propagating in the beamtube, it is necessary reduce the stray light by absorption on the beamtube walls. Many optically absorbing coatings tend to increase the surface area of the tube  and could have poor outgassing properties.

Research is needed into coatings or surface treatment of the inner wall of the aluminum tube that satisfies both the optical and vacuum requirements.

Acoustic and mechanical noise: Need to think about the motion of the structures driven by thermal expansion and seismic vibrations. Need to avoid stick/slip noise as the temperature changes that can communicate to the structures that support the mirrors. Need to avoid poorly damped oscillations in the inner system in the frequency band of the gravitational wave observations (5Hz to 5kHz). Depending on current optical calculations, it may even be most economical to (crudely) vibration isolate the inner tube from seismic and acoustic noise.

Assembly:  A key element for a nested system using the concept described here is to learn how to fabricate and assemble the inner aluminum system. Bonding does not need to be done to the usual UHV leak requirements. It is worth looking at techniques such as roll bonding, laser welding, ultrasonic pressure welding  - techniques that can be done in the field as part of assembly. One can imagine an assembly system starting with a machine (much like CB&I used in LIGO) to convert coil stock to tubing by spiral joining followed by a sheet metal forming press to make the V thermal relief bends. The formed tube cleaned, coated, and dried in a final stage of the movable assembly system.  A technique needs to be developed to join sections of the aluminum tube with the valves that can still seal around a step discontinuity in the tube wall (places where the sheet metal overlaps). Given the overall cost of making a cosmic explorer, an investment of several millions dollars into such an assembly system would be cost effective. $3M is assumed for this in the preliminary budget.

Metrology: Optical column density gauges using laser beams with wavelengths specific to the individual gases would be a good way to determine the amount of residual gas left in the tubes. Tunable pulsed UV lasers tuned to the strong molecular electronic lines could make the species determination while total absorption on and off resonance could provide column density measurements.  Timing of the scattered light could also provide some information of the location of leaks in the system.

Over all tests: Once concepts have survived more critical engineering study, the surviving ideas need to be tested in prototype. This was absolutely critical in the development of the beamtube vacuum system for initial LIGO which was tested twice. Once in a small 20 meter prototype and then again at CB&I in a full fledged prototype using the dimensions and practices to be employed in the field construction. Unless there are good reasons developed at the workshop otherwise,  a prototype commercial standard 48” extruded steel tube should be tested for leaks and outgassing followed by the insertion of a thin wall tube to make a nested system allowing an inner system bakeout.

References

Historical Pipeline Construction  Cost Analysis

Z. Rui, P.A. Metz, D. Reynolds, G,Chen , X. Zhou International Journal of Oil, Gas and Coal Technology V4 #3 244 (2011)

“Assessing and Bench Marking the Construction Cost of Tunnels”  A. Bernardos , Mark Diederichs, C. Paraskevopoulou GEOMontreal  October (2013)

Residual Gas in the LIGO Beam Tubes

R. Weiss  (2003)

DCC LIGO G030701

Reanalysis of average pressure in the beamtube as a function of water injected at the ends

R. Weiss (2008)

DCC  LIGO T080330-00